Pointers

Question: If those memory addresses are so fundamental for programming, can they be represented in the C language? The answer is YES - addresses can be stored inside special variables called pointers. Let’s see an example:

int *p_int;

p_int is a pointer to integers. In other words, p_int is a variable that can hold memory addresses of integer variables. If so, p_int can hold the address of the counter variable:

p_int = &counter;

It is also useful to get the value stored in the memory location pointed by p_int. This is call pointer dereferencing. We use the indirection operator (*) as the prefix to dereference a pointer:

*(pointer_name)

Therefore the while loop can become:

while (*(p_int) < 10) { (*p_int) ++; }

*(p_int) means the value at the address currently stored in the p_int poiner, which, in this case, is the value of the counter variable.

Now lets add *p_int to the Expression tab and look at its value. Indeed *p_int is an alias for counter.

Finally lets see the incredible power we have with pointers:

p_int = (int *)0x20000010;

*p_int = 0xDEADBEEF;

Examine the value stored in 0x20000010 in the Memory tab (you can also try with a misaligned address i.e. 0x20000012)

Exercise 1:

In C, create an integer variable x with an initial value of 10. Use a pointer to reference and change the value of x to 20. Dereference the pointer to print the new value of x.

Solution:

int x = 10;

int *ptr = &x;  // Reference

*ptr = 20;       // Dereference and modify

Exercise 2:

Given the integer array int arr[3] = {5, 10, 15}; in C:

Use pointer arithmetic to calculate the sum of the first and second elements.

Store the result in a new variable sum.

Solution:

int arr[3] = {5, 10, 15};

int *ptr = arr;

int sum = *ptr + *(ptr + 1);  // Sum of first two elements

Exercise 3:

In C, define an array int nums[4] = {2, 4, 6, 8};.

Use a pointer to iterate through the array and multiply each element by 2.

Solution:

int nums[4] = {2, 4, 6, 8};

int *ptr = nums;

for (int i = 0; i < 4; i++) {

*(ptr + i) *= 2;  // Multiply each element by 2

}

Exercise 4:

In C, write a function void swap(int *a, int *b) to swap two integers using call by reference.

Test the function with x = 7 and y = 9.

Solution:

void swap(int *a, int *b) {

int temp = *a;

*a = *b;

*b = temp;

}

int x = 7, y = 9;

swap(&x, &y);

ARM ISA

Exercise 1: Simple arithmetic

Set R1 = 0x30 and R2 = 0x1A. Then write ARM assembly code to compute the following:

- R3 = (R1 + R2) × 4

- R4 = R1 ⊕ R2

- R5 = R2 >> 3

- R6 = R1 × 8 (using only shifts)

What is the result?

Solution

MOV r1, #0x30

MOV r2, #0x1a

ADD r3, r1, r2      // r3 = r1 + r2

LSL r3, r3, #2      // r3 = r3 * 4

EOR r4, r1, r2      // r4 = r1 XOR r2

LSR r5, r2, #3      // r5 = r2 >> 3

LSL r6, r1, #3      // r6 = r1 * 8

Exercise 2: Data movement

Write ARM assembly code to:

- Load the value 0x1234 into r0.

- Store the value in memory at address 0x20000000.

- Load it back from memory into r1.

- Modify the code to increment the value in memory by 1

Solution:

MOV r0, #0x1234      // Load value into r0

LDR r2, =0x20000000  // Load memory address

STR r0, [r2]         // Store value

LDR r1, [r2]         // Load value back into r1

ADD r1, r1, #1       // Increment value

STR r1, [r2]         // Store incremented value

Exercise 3: Simple arithmetic

Store the following array of integers in memory starting at address 0x20000000: [0x9, 0x25, 0x46].

Then, for this array perform the following steps:

- Write ARM assembly code to calculate the sum of the first three elements.

- Multiply the second element by 3 and store it back in memory.

Solution:

// Array base address: 0x20000000

LDR R0, =0x20000000  // Load base address of array into R0

// Store array in memory

MOV R1, #0x9

STR R1, [R0]

MOV R1, #0x25

STR R1, [R0, #4]

MOV R1, #0x46

STR R1, [R0, #8]

// 1. Sum the first three elements

LDR R1, [R0]          // Load first element into R1

LDR R2, [R0, #4]      // Load second element into R2

LDR R3, [R0, #8]      // Load third element into R3

ADD R4, R1, R2        // Add first and second elements

ADD R4, R4, R3        // Add third element

// 2. Multiply the second element by 3 and store it back

LDR R1, [R0, #4]      // Load second element into R1

MOV R2, #0x3

MUL R1, R1, R2        // R1 = R1 * 3

STR R1, [R0, #4]      // Store result back at second element's address