Exercise 1:

Define two new functions, function1 and function2 that accept an integer as an argument. Call them one after the other from ASM_Function, making sure that the callee returns every time to the caller.

- function1 should increase the value of the argument by 5

- function2 should decrease it by 2

Exercise 2: Compare numbers

Write an ARM assembly program that compares two signed integers and branches based on the result. If the first integer is greater than the second, the program should set the result to 1. Otherwise, it should set the result to 0.

Solution:

compare_numbers:

// Inputs: r0 = first number, r1 = second number

cmp r0, r1         // Compare r0 and r1

bgt greater       // Branch to "greater" if r0 > r1

movs r0, #0      // Set result to 0 (r0 <= r1 case)

bx lr                   // Return

greater:

movs r0, #1      // Set result to 1 (r0 > r1 case)

bx lr                   // Return

Exercise 3: Loop with Branching

Write a program that sums all integers from 1 to a given number n > 1. Implement the loop using branching instructions.

Solution:

sum_numbers:

// Input: r0 = n (the upper limit)

movs r1, #0        // Initialize sum in r1

movs r2, #1        // Initialize counter i = 1

loop:

cmp r2, r0         // Compare counter i (r2) with n (r0)

bgt end_loop       // If i > n, break out of the loop

adds r1, r1, r2    // sum = sum + i

adds r2, r2, #1    // i = i + 1

b loop             // Repeat the loop

end_loop:

mov r0, r1         // Store the result in r0

bx lr              // Return

Exercise 4: Nested Branches (Maximum of Three Numbers)

Write an ARM assembly program that finds the maximum of three unsigned integers. The numbers are passed in registers r0, r1, and r2, and the result (maximum value) is returned in r0.

Solution:

max_of_three:

// Inputs: r0 = first number, r1 = second number, r2 = third number

cmp r0, r1         // Compare first two numbers (r0, r1)

bgt check_third     // If r0 > r1, check with third number

mov r0, r1         // Otherwise, set r0 = r1 (max so far)

check_third:

cmp r0, r2         // Compare max(r0) with r2

bgt end            // If r0 > r2, we're done

mov r0, r2         // Otherwise, set r0 = r2 (max is r2)

end:

bx lr              // Return the result

Exercise 5: if-else control flow

Write a program that takes an integer as an input and returns 1 if the input is positive, -1 if the input is negative and 0 if the input is zero

Solution:

if_else_example:

// Input: r0 = number

cmp r0, #0            // Compare input with 0

beq return_zero       // If input == 0, go to return_zero

bmi return_negative   // If input < 0 (negative), go to return_negative

// Input is positive

movs r0, #10          // Return 10 for positive input

bx lr                 // Return

return_negative:

movs r0, #-10         // Return -10 for negative input

bx lr                 // Return

return_zero:

movs r0, #0           // Return 0 for zero input

bx lr                 // Return

Bonus: exercise 6: Write a program that computes the n-th Fibonacci number

Solution

fibonacci:

// Input: r0 = n (the Fibonacci number to compute)

cmp r0, #1            // Compare n with 1

ble done              // If n <= 1, return n (either 0 or 1)

// Initialize first two Fibonacci numbers

movs r1, #0           // r1 = fib(0) = 0

movs r2, #1           // r2 = fib(1) = 1

subs r0, r0, #1       // r0 = n - 1 (remaining iterations)

fib_loop:

subs r0, r0, #1       // Decrement n (r0 = r0 - 1)

beq done              // If n == 0, we are done

adds r3, r1, r2       // r3 = r1 + r2 (fib(i-2) + fib(i-1))

mov r1, r2            // r1 = fib(i-1)

mov r2, r3            // r2 = fib(i)

b fib_loop            // Repeat the loop

done:

mov r0, r2            // r0 = nth Fibonacci number

bx lr                 // Return

Explanation:

- Base Case: If n <= 1, it directly returns the value n because fib(0) = 0 and fib(1) = 1.

- Loop: For n > 1, the program initializes r1 to fib(0) and r2 to fib(1). It then iteratively adds the previous two numbers in the sequence until it computes the n-th Fibonacci number.

- Final Return: The result is stored in r0, which holds the n-th Fibonacci number.